LAB 4

OUTLINE

I. Input Ports (Transition)

II. Output Ports (Capacitive Load)

Design Schematic

I. Input Ports (Transition)

| DESIGN SPECIFICATION | COMMAND AND COMMAND EXPLANATION |
| --- | --- |
| 1. Specify a driving cell called bufbd1 on all inputs, except clk as well as Cin* | Command: set_driving_cell -lib_cell bufbd1 [remove_from_collection [all_inputs] [get_ports {clk Cin1[*] Cin2[*]}]] Explantion: + set_driving_cell assigns a driving strength model to all input ports (except clk, Cin1, and Cin2) using the standard cell bufbd1. + The option -lib_cell bufbd1 specifies that the signal source behaves like cell bufbd1, defining its drive capability and output slew. + [remove_from_collection [all_inputs] [get_ports {clk Cin1[*] Cin2[*]}]] ensures the driving cell is applied to all inputs except the specified ports. + Ensures accurate modeling of input drive strength for synthesis and timing analysis. |
| 2. The Cin* ports are chip-level inputs and have a 120 ps maximum input transition. | Command: set_input_transition -max 0.12 [get_ports {Cin1[*] Cin2[*]}] Explantion: + set_input_transition constrains the maximum input transition time (slew) for the specified ports. + -max 0.12 represents 0.12 ns, which is equivalent to 120 ps, the maximum transition allowed for Cin1 and Cin2. + [get_ports {Cin1[*] Cin2[*]}] selects all bits of the Cin buses. + Ensures proper modeling of input signal transitions for top-level (chip-level) input ports. |

II. Output Ports (Capacitive Load)

| DESIGN SPECIFICATION | COMMAND AND COMMAND EXPLANATION |
| --- | --- |
| 1. All outputs, except Cout, drive a maximum load equivalent to 2 times the capacitance of the “I” pin of the cell bufbd7. | Command: set_load [expr 2 * [load_of cb13fs120_tsmc_max/bufbd7/I]] [remove_from_collection [all_outputs] [get_ports {Cout[*]}]] Explantion: + set_load defines the output capacitive load for timing calculation. + [expr 2 * [load_of cb13fs120_tsmc_max/bufbd7/I]] doubles the capacitance of the input pin “I” of cell bufbd7 to model higher load conditions. + [remove_from_collection [all_outputs] [get_ports {Cout[*]}]] applies this load to all outputs except Cout. + Ensures realistic output loading for accurate synthesis and timing. |
| 2. The Cout port drives a maximum load of 25 fF. | Command: set_load 0.025 [get_ports {Cout[*]}] Explantion: + set_load defines the fixed capacitive load for the output port Cout. + 0.025 represents a load value of 0.025 picofarads (pF), which is equivalent to 25 femtofarads (fF). + [get_ports {Cout[*]}] selects the entire output bus Cout. + Ensures that the Cout port meets its specified load requirement of 0.025 pF, providing accurate timing and power modeling during synthesis. |