LAB 5: DC ULTRA SYNTHESIS TECHNIQUES

OUTLINE

I. Design Specification

II. Lab Instructions

Task 1. Synthesis of RTL design containing pipelines

Task 2. OPTIONAL: Formal Verification

Design Schematic

I. Design Specification

| DESIGN SPECIFICATION | COMMAND AND COMMAND EXPLANATION |
| --- | --- |
| 1. The I/O constraints are estimates and have been conservatively constrained | Command: source ./scripts/STOTO.con Explantion: + The source command reads and applies all timing and environmental constraints defined in the external file STOTO.con. + This file already includes the clock, uncertainty, input delay, output delay, and load definitions required for synthesis. + Using a constraint file ensures all design I/O conditions are applied consistently and conservatively, preventing early timing violations during synthesis. |
| 2. The final compiled design must meet setup timing on all internal register-to-register paths | Command: group_path –name INPUTS –from [all_inputs] group_path –name OUTPUTS –to [all_outputs] group_path –name COMBO –from [all_inputs] –to [all_outputs] group_path –name clk –critical_range [expr 2.1 * 0.1] -weight 5 Explantion for each command: Command 1: group_path –name INPUTS –from [all_inputs] + Creates a timing group called INPUTS for all paths starting from the input ports. + This helps Design Compiler analyze and optimize timing between inputs and internal registers. Command 2: group_path -name OUTPUTS -to [all_outputs] + Defines a timing group OUTPUTS that includes all paths ending at the output ports. + Allows the tool to check register-to-output setup timing and ensure proper delay balancing. Command 3: group_path –name COMBO –from [all_inputs] –to [all_outputs] + Defines a COMBO path group that includes all input-to-output combinational paths. + Helps in reporting and analyzing any direct combinational timing violations. Command 4: group_path –name clk –critical_range [expr 2.1 * 0.1] -weight 5 + Creates a clk timing group to represent all register-to-register paths under the main clock. + -critical_range [expr 2.1 * 0.1] defines the critical slack range as 10% of the 2.1 ns clock period (0.21 ns). + -weight 5 increases optimization priority for paths near the clock’s critical range, ensuring setup timing closure for the main sequential logic. |
| 3. The INPUT block hierarchy should be preserved to facilitate post-synthesis verification | Command: set_ungroup [get_designs INPUT] false Explantion: + The set_ungroup command prevents Design Compiler from flattening the INPUT block during synthesis. + This maintains the hierarchical structure between RTL and gate-level design. + Keeping hierarchy helps in debugging and verifying the design after synthesis, as the module boundaries remain intact. |
| 4. The PIPELINE block contains a “pure” pipelined design | Command: set_optimize_registers true -design [get_designs PIPELINE] Explantion: + The set_optimize_registers command allows Design Compiler to perform register balancing and retiming inside the PIPELINE block. + It ensures that registers are relocated internally for improved timing without changing functional pipeline behavior. + This keeps the block as a “pure” pipelined structure with proper register separation between logic stages. |
| 5. The output (POUT[9:0]) of PIPELINE must remain registered | Command: set_dont_retime [get_cells I_MIDDLE/I_PIPELINE/z2_reg*] true Explantion: + The set_dont_retime command prevents Design Compiler from moving or removing registers associated with the z2_reg outputs of the PIPELINE block. + This ensures that the block’s outputs remain fully registered, preserving the integrity of pipelined output timing and functionality. |
| 6. The logic positions of registers in the instance or cell I_DONT_PIPELINE are fixed and cannot be modified | Command: set_dont_touch [get_cells I_MIDDLE/I_DONT_PIPELINE/*] Explantion: + The set_dont_touch command disables any optimization, restructuring, or retiming within the I_DONT_PIPELINE block. + This preserves both logic and register positions exactly as designed, ensuring consistency with previously verified functionality. |
| 7. DRC violations should be fixed unless they prevent setup timing constraints from being met | Command: Set_cost_priority -delay Explantion: + The set_cost_priority command controls the optimization goal of Design Compiler. + Using -delay prioritizes timing optimization (setup closure) over fixing minor DRC issues such as transition or capacitance violations. + This ensures that setup timing remains the top priority during synthesis. |
| 8. The logic position of registers may be modified to improve timing, except where expressly prohibited or handled differently by above specs. | Command: compile_ultra -spg -retime Explantion: + The -retime option enables adaptive retiming, allowing automatic relocation of registers to improve setup timing. + The -spg option (Synopsys Physical Guidance) activates physically-aware synthesis, using placement estimation to achieve better timing correlation with layout. + Together, these options allow Design Compiler to perform intelligent retiming while respecting the set_dont_retime and set_dont_touch constraints applied earlier. |
| 9. Scan insertion will be performed by the “Testgroup” after the design has met these specifications | Command: compile_ultra -scan Explantion: + The -scan option prepares the design for test-ready synthesis by replacing flip-flops with scan-capable versions, without connecting scan chains. + This makes the design DFT-ready for the Testgroup to complete scan insertion later. |

II. Lab Instructions

| TASK 1. SYNTHESIS OF RTL DESIGN CONTAINING PIPELINES |  |
| --- | --- |
| QUESTION | ANSWER |
| Question 4: What compile_ultra options will you apply? | Command: compile_ultra -spg -retime + The -retime option enables adaptive retiming, allowing Design Compiler to automatically move or balance registers to improve setup timing. + The -spg (Synopsys Physical Guidance) option performs physically aware synthesis, which uses placement estimation to achieve timing results closer to post-layout. + These options ensure that register timing is optimized while respecting constraints from previous specifications such as set_dont_retime and set_dont_touch. + Together, they improve timing closure for internal register-to-register paths while maintaining pipeline structure integrity. |
| Question 5: Is the design well partitioned for synthesis? Explain. | + The design is not well partitioned for synthesis. + Logic optimization is restricted at hierarchical boundaries, which prevents cross-module optimization between these sub-designs: GLUE → ARITH ARITH → RANDOM RANDOM → OUTPUT + Because of this hierarchical isolation, optimization cannot propagate across these interfaces. + To achieve better synthesis results, these sub-designs must be ungrouped or flattened where timing is critical. |
| Question 6: Which sub-designs would need to be ungrouped to obtain ideal partitioning for synthesis? | + That sub-designs is : MIDDLE, DONT_PIPELINE, GLUE, ARITH, RANDOM, OUTPUT + Ungrouping these sub-designs allows cross-boundary optimization, enabling the synthesis tool to: - Move logic or registers across boundaries. - Merge combinational logic between modules. - Balance timing paths for improved setup closure. + This eliminates hierarchy restrictions that slow down or limit optimization. |
| Question 16: Were ALL registers in the INPUT sub-design affected by adaptive retiming? | + The “-retime” option in the compile_ultra command enables register movement. To verify whether the INPUT sub-design was affected, use: get_cells I_IN/*reg* + Registers modified by adaptive retiming are automatically renamed with the prefix “R”, followed by a numerical identifier (R##). |
| Question 17: What are the next suggested steps if there are still violations after the initial compile? | Command & Steps: +) Prioritize critical paths: group_path -weight -critical Increases focus on near-critical paths with small slack violations. But i already set critical range at 10% of clock period in the beginning. +) Perform an incremental compile: compile_ultra -scan -retime -incremental -scan: Keeps DFT-ready synthesis active. -retime: Continues register movement to improve timing. -incremental: Recompiles only changed portions, saving time and maintaining stable results on non-violating paths. Explanation: These commands refine optimization on the most timing-critical sections of the design without re-synthesizing the entire chip. |
| Question 18: Based on Design Specification #1 and #2, does it make sense to perform these steps? | + Yes, it is necessary to load the constraint file and create group paths, while also assigning a critical range and weight to the paths that require timing improvement. |
| TASK 2. OPTIONAL: FORMAL VERIFICATION |  |
| Question 21: Which file contains SVF information written out by DC? | That file is: STOTO.svf + The file STOTO.svf stores SVF (Setup Verification for Formality) information generated by Design Compiler. + The set_svf command is used before synthesis to specify this file name, ensuring that all transformations affecting logic equivalence are recorded. + This file allows Formality to reproduce synthesis transformations and confirm logical equivalence between RTL and gate-level designs. |
| Question 22: Which files are the “Reference” and “Implementation” design files that are being verified for functional equivalence? | That file is: rtl/STOTO.v and mapped/stoto.ddc + rtl/STOTO.v: This file represents the original behavioral model before synthesis. + stoto.ddc: This file is produced after synthesis using compile_ultra, representing the optimized design in Design Compiler format. + Both files are then compared in Formality to verify that synthesis transformations did not alter functionality. |
| Question 23: Does verification succeed or fail? |  |