LAB 9: OPTIONAL LAB

Constraints: Complex Design Considerations

OUTLINE

I. Clock Spec

II. Input Port Delays

III. Output Port Delays

IV. External Load, Inputs Ports

Design Schematic

I. Clock Spec

| DESIGN SPECIFICATION | COMMAND AND COMMAND EXPLANATION |
| --- | --- |
| 1. Rename the clock from “clk” to “my_clk” 2. Change the duty cycle to 40% active high, with zero offset. | Command: create_clock -name my_clk -period 3.0 -waveform {0 1.2} [get_ports clk] Explantion: + create_clock defines a new clock named my_clk on the same input port. + -period 3.0 assigns a 3 ns period (333.33 MHz) clock, consistent with the design frequency. + -waveform {0 1.2} defines the clock waveform: - The clock rises at 0 ns and falls at 1.2 ns. - Clock high time = 1.2 ns, total period = 3 ns → (1.2 ÷ 3.0) × 100% = 40% duty cycle. + “Zero offset” means the first rising edge starts at 0 ns. + This ensures that all subsequent constraints refer to the new clock name my_clk. + Without redefining the name, DC would still reference the old clk clock, causing mismatched timing relationships. |

II. Input Port Delays

| DESIGN SPECIFICATION | COMMAND AND COMMAND EXPLANATION |
| --- | --- |
| 1. The sel port data is being supplied by an additional register, F4. The data at sel arrives no later than 420ps after the negative edge-triggered launching clock edge of F4. 2. The clock pin at F4 sees a 600ps total latency (source + network), which is different than the total latency of my_clk. Use the “latencies included” method to incorporate this latency. | Command: set_input_delay -max 1.02 -clock my_clk -clock_fall -add_delay -source_latency_included -network_latency_included [get_ports sel] Explantion: + The value -max 1.02 is calculated as follows: Input delay = (tck2q + tcomb) + ttotal latency = 420 + 600 = 1020 ps = 1.02 ns + set_input_delay defines the delay between the launching clock edge and the arrival of data at the design’s input pin. + -max 1.02 sets the maximum input delay to 1.02 ns. + -clock my_clk associates this delay with the timing of my_clk. + -clock_fall specifies that the data is launched on the falling edge (negative-edge triggered register F4). + -source_latency_included -network_latency_included tells DC that the external environment (F4) already includes clock propagation effects. This prevents double-counting of latency and ensures correct clock alignment between internal and external timing models. + [get_ports sel] targets the exact input port. |

III. Output Port Delays

| DESIGN SPECIFICATION | COMMAND AND COMMAND EXPLANATION |
| --- | --- |
| 1. The out1 port data is captured by an additional register, F5. The data at out1 must arrive no later than 260ps before F5’s negative edge-triggered clock edge. 2. The clock pin at F5 sees a network latency of 500ps (its source latency is the same as that of my_clk). Use the “latencies included” method to incorporate this latency. | Command: set_output_delay -max -0.94 -clock my_clk -clock_fall -add_delay -source_latency_included -network_latency_included [get_ports out1*] Explantion: - Because of this is type half cycle path, so we will check setup slack at edge as above image. - The -max -0.94 is calculated as follows: Setup Slack = Trequire – Tarrival ≥ 0 = (T + tcapture – tsetup – tcomb2) – (tck2q + tcomb1 + tlaunch) ≥ 0 = (T + Tout) – (tck2q + tcomb1 + tlaunch) ≥ 0 = 1.2 + Tout – 0.26 ≥ 0 → Tout ≥ -0.94 + set_output_delay defines how long before the capture clock edge the output data must be valid for correct setup timing. + -clock my_clk references the timing of the capturing clock (F5). + -clock_fall specifies that data is captured at the falling edge of F5’s clock. + -add_delay ensures compatibility if more output constraints are later applied. + [get_ports out1] targets the exact output port. + This command models the setup relationship between the last register inside MY_DESIGN and the external capturing register F5. + It helps DC perform accurate setup slack analysis, considering both internal logic delay and external clock propagation. |

IV. External Load, Inputs Ports

| DESIGN SPECIFICATION | COMMAND AND COMMAND EXPLANATION |
| --- | --- |
| 1. Each input port (except clk) fans out to 2 other sub-blocks, which each drive the equivalent of 3 bufbd1 (input pin I) buffers internally. Model this external capacitive loading on the inputs. | Command: set_driving_cell -max -lib_cell bufbd1 [remove_from_collection [all_inputs] [get_ports clk]] Explantion: + The set_driving_cell command assigns a driving cell model for input ports. + -lib_cell bufbd1 defines that each input port is driven by the standard-cell buffer bufbd1 from the library. + -max specifies that this setting applies to the worst-case (maximum) timing condition. + [remove_from_collection [all_inputs] [get_ports clk]] ensures that the clock input (clk) is excluded from this setting. + This command helps DC model realistic input transition (slew) values based on the drive strength of the external buffer. |